UIOX Project · 02_FwHal Hardware Reference

# Hardware Description & Pin Layout Manual

All peripherals in 02_FwHal/include — Registers · Signal Pins · Protocols · MMIO Maps

This document describes every hardware peripheral covered by the UIOX Firmware HAL (02_FwHal/include/). For each peripheral it provides: hardware description, register map, signal/pin layout, electrical characteristics, MMIO base addresses, IRQ assignments, and the driver API defined in the corresponding header file.

# 1 · Platform Overview

The UIOX firmware HAL supports four hardware targets with a common driver interface. All peripheral base addresses, IRQ numbers, and pin assignments are selected at compile time by the ARCH_* flag injected by the root Makefile.

| Peripheral | ARM64 MMIO Base | ARM32 MMIO Base | x86-64 I/O or MMIO | RISC-V MMIO Base | IRQ |
| --- | --- | --- | --- | --- | --- |
| UART (PL011 / 16550) | 0x09000000 | 0x10009000 | Port 0x3F8 (COM1) | 0x10000000 | 33 / 44 / 4 / 10 |
| SPI (PL022 SSP) | 0x09050000 | 0x10115000 | ICH SPI MMIO | 0x10004000 | — |
| Timer SP804 / PIT | 0x09010000 | 0x10011000 | Port 0x40 (PIT) | CLINT 0x02000000 | 27 / 36 / 0 / 5 |
| GIC-400 Distributor | 0x08000000 | 0x08000000 | LAPIC 0xFEE00000 | PLIC 0x0C000000 | — |
| VirtIO Block | 0x0A000000 | 0x0A000000 | PCIe ECAM | 0x10001000 | 48 / 48 / 46 / 1 |
| PCIe ECAM | 0x3F000000 | — | 0xB0000000 | 0x30000000 | 32+ |
| USB xHCI | PCIe BAR0 | PCIe BAR0 | PCIe BAR0 | PCIe BAR0 | PCIe MSI |
| SD/SDIO Host | 0xFE340000 | 0x10005000 | — | 0x10005000 | 22 |
| NVMe (PCIe BAR0) | PCIe | PCIe | PCIe | PCIe | PCIe MSI |
| Watchdog SP805 | 0x09060000 | 0x10010000 | iTCO MMIO | 0x10006000 | 32 |
| Ethernet VirtIO/SMSC | VirtIO | 0x10120000 | PCIe | VirtIO | VirtIO/IRQ |
| SPI Sensor Bus | 0x09050000 | 0x10115000 | — | 0x10004000 | — |

# 2 · UART — Universal Asynchronous Receiver / Transmitter

Header: 02_FwHal/include/uiox_fw_uart.h

UIOX supports two UART hardware IP cores: ARM PrimeCell PL011 (ARM64/ARM32) and 16550A-compatible (x86-64, RISC-V). Both provide full-duplex asynchronous serial communication with programmable baud rate, data format, and hardware flow control.

## 2.1  Hardware Description

| Parameter | PL011 (ARM) | 16550A (x86 / RISC-V) |
| --- | --- | --- |
| Interface | AMBA APB slave | ISA / AXI bus slave |
| Data width | 8 / 7 / 6 / 5 bits | 8 / 7 / 6 / 5 bits |
| Stop bits | 1 or 2 | 1 or 2 |
| Parity | None / Odd / Even / Stick | None / Odd / Even |
| Baud rate | Fractional divisor (IBRD+FBRD) | Divisor latch (DLL+DLM) |
| FIFO depth | 16 bytes TX + 16 bytes RX | 16 bytes TX + 16 bytes RX |
| Flow control | RTS/CTS hardware | RTS/CTS hardware |
| Reference clock | 24 MHz (QEMU default) | 1.8432 MHz ÷ divisor |
| IRQ trigger | Level (IMSC mask register) | Level (IER register) |

## 2.2  Signal / Pin Layout

| Pin Name | Direction | Description | Active Level |
| --- | --- | --- | --- |
| TXD | Output | Transmitted serial data | High idle |
| RXD | Input | Received serial data | High idle |
| RTS | Output | Request To Send — asserted when FIFO has space | Low |
| CTS | Input | Clear To Send — remote ready to receive | Low |
| DTR | Output | Data Terminal Ready (modem control) | Low |
| DSR | Input | Data Set Ready (modem status) | Low |
| DCD | Input | Data Carrier Detect | Low |
| RI | Input | Ring Indicator | Low |
| nRESET | Input | Hardware reset (active low) | Low |
| PCLK | Input | APB peripheral clock (PL011) | Rising edge |
| UARTCLK | Input | Reference clock for baud generation | Rising edge |
| INTR | Output | Interrupt output to GIC/APIC/PLIC | High |

## 2.3  PL011 Register Map

| Offset | Register | R/W | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | UARTDR | R/W | 0x000 | Data Register — TX write / RX read (bits[7:0]=data, bits[11:8]=error flags on read) |
| 0x004 | UARTRSR | R/W | 0x000 | Receive Status / Error Clear — OE/BE/PE/FE flags on read; write any to clear |
| 0x018 | UARTFR | R | 0x090 | Flag Register — TXFF[5]=TX FIFO full, RXFE[4]=RX FIFO empty, BUSY[3]=TX active |
| 0x020 | UARTILPR | R/W | 0x000 | IrDA Low-Power Counter Register |
| 0x024 | UARTIBRD | R/W | 0x000 | Integer Baud Rate Divisor — IBRD = UARTCLK / (16 × baud) |
| 0x028 | UARTFBRD | R/W | 0x000 | Fractional Baud Rate Divisor — FBRD = round(frac × 64) |
| 0x02C | UARTLCR_H | R/W | 0x000 | Line Control — WLEN[6:5]=word len, FEN[4]=FIFO enable, STP2[3]=2 stop, PEN[1]=parity |
| 0x030 | UARTCR | R/W | 0x300 | Control — CTSEN[15], RTSEN[14], RXE[9]=RX enable, TXE[8]=TX enable, UARTEN[0] |
| 0x034 | UARTIFLS | R/W | 0x012 | FIFO Level Select — RX/TX interrupt thresholds (1/8, 1/4, 1/2, 3/4, 7/8) |
| 0x038 | UARTIMSC | R/W | 0x000 | Interrupt Mask — RXIM[4]=RX mask, TXIM[5]=TX mask, RTIM[6]=RX timeout |
| 0x03C | UARTRIS | R | 0x000 | Raw Interrupt Status |
| 0x040 | UARTMIS | R | 0x000 | Masked Interrupt Status (RIS AND IMSC) |
| 0x044 | UARTICR | W | — | Interrupt Clear Register — write 1 to clear each bit |

## 2.4  16550A Register Map

| Port Offset | DLAB=0 Name | DLAB=1 Name | R/W | Description |
| --- | --- | --- | --- | --- |
| Base+0 | RBR (read) | DLL | R/W | Receive Buffer / TX Holding / Divisor Latch Low |
| Base+1 | IER | DLM | R/W | Interrupt Enable / Divisor Latch High |
| Base+2 | IIR (read) | FCR (write) | R/W | Interrupt ID / FIFO Control |
| Base+3 | LCR | LCR | R/W | Line Control — WLS[1:0], STB[2], PEN[3], DLAB[7] |
| Base+4 | MCR | MCR | R/W | Modem Control — DTR[0], RTS[1], OUT1[2], OUT2[3] |
| Base+5 | LSR | LSR | R | Line Status — DR[0]=data ready, THRE[5]=TX holding empty, TEMT[6]=TX empty |
| Base+6 | MSR | MSR | R | Modem Status — CTS[4], DSR[5], RI[6], DCD[7] |
| Base+7 | SCR | SCR | R/W | Scratch Register (no hardware function) |

## 2.5  Baud Rate Calculation

PL011 (24 MHz clock, 115200 baud):

IBRD = 24000000 / (16 × 115200) = 13

FBRD = round((0.0208... × 64)) = 1

16550A (1.8432 MHz clock, 115200 baud):

Divisor = 1843200 / (16 × 115200) = 1  → DLL=0x01, DLM=0x00

# 3 · SPI — Serial Peripheral Interface (PL022 SSP)

Header: 02_FwHal/include/uiox_fw_spi.h

The ARM PrimeCell PL022 Synchronous Serial Port (SSP) provides SPI, SSP (TI), and Microwire frame formats. UIOX uses SPI mode exclusively. Connected devices include NOR flash, TFT displays (ST7789/ILI9341), ALS sensors (TSL2591), IMU (BMI270), and SD cards in SPI fallback mode.

## 3.1  Signal / Pin Layout

| Pin | Direction | SPI Name | Description |
| --- | --- | --- | --- |
| SPICLK | Output (master) | SCLK | Serial clock — idle level set by CPOL |
| SPITXD | Output (master) | MOSI | Master-Out Slave-In data |
| SPIRXD | Input (master) | MISO | Master-In Slave-Out data |
| nSS[0] | Output (master) | CS0 | Chip Select 0 — active low |
| nSS[1] | Output (master) | CS1 | Chip Select 1 |
| nSS[2] | Output (master) | CS2 | Chip Select 2 |
| nSS[3] | Output (master) | CS3 | Chip Select 3 |
| SSPCLK | Input | PCLK | APB peripheral bus clock |
| nSSPRST | Input | RESET | Active-low hardware reset |
| SSPINTR | Output | IRQ | Interrupt to GIC/PLIC |
| SSPDMARXBREQ | Output | DMA-RX | DMA burst request RX FIFO |
| SSPDMATXBREQ | Output | DMA-TX | DMA burst request TX FIFO |

## 3.2  SPI Modes (CPOL + CPHA)

| Mode | CPOL | CPHA | Clock Idle | Data Sampled On | UIOX Enum |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Low | Rising edge | UIOX_SPI_MODE_0 |
| 1 | 0 | 1 | Low | Falling edge | UIOX_SPI_MODE_1 |
| 2 | 1 | 0 | High | Falling edge | UIOX_SPI_MODE_2 |
| 3 | 1 | 1 | High | Rising edge | UIOX_SPI_MODE_3 |

## 3.3  PL022 Register Map

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x000 | SSPCR0 | R/W | Control 0 — DSS[3:0] data size, FRF[5:4] frame format, SPO[6] CPOL, SPH[7] CPHA, SCR[15:8] clock divider |
| 0x004 | SSPCR1 | R/W | Control 1 — LBM[0] loopback, SSE[1] enable, MS[2] master/slave, SOD[3] slave output disable |
| 0x008 | SSPDR | R/W | Data Register — 16-bit TX write / RX read (FIFO-backed) |
| 0x00C | SSPSR | R | Status — TFE[0]=TX FIFO empty, TNF[1]=TX not full, RNE[2]=RX not empty, RFF[3]=RX FIFO full, BSY[4]=busy |
| 0x010 | SSPCPSR | R/W | Clock Prescale — CPSDVSR[7:0] must be even (2–254); fSSPCLK = PCLK / (CPSDVSR × (1+SCR)) |
| 0x014 | SSPIMSC | R/W | Interrupt Mask — RORIM[0], RTIM[1], RXIM[2], TXIM[3] |
| 0x018 | SSPRIS | R | Raw Interrupt Status |
| 0x01C | SSPMIS | R | Masked Interrupt Status |
| 0x020 | SSPICR | W | Interrupt Clear — write 1 to clear RORIC[0] and RTIC[1] |
| 0x024 | SSPDMACR | R/W | DMA Control — RXDMAE[0], TXDMAE[1] |

## 3.4  Clock Frequency Calculation

f_SPI = PCLK / (CPSDVSR × (1 + SCR))

Example: PCLK=24MHz, CPSDVSR=4, SCR=5 → f_SPI = 24000000/(4×6) = 1 MHz

# 4 · Timer — SP804 / PIT 8254 / ARM Generic Timer / RISC-V CLINT

Header: 02_FwHal/include/uiox_fw_timer.h

## 4.1  Hardware Description

| Property | SP804 (ARM) | PIT 8254 (x86) | ARM Generic Timer | RISC-V CLINT mtime |
| --- | --- | --- | --- | --- |
| Type | Down-counter, 32-bit | Down-counter, 16-bit | Up-counter, 64-bit | Up-counter, 64-bit |
| Clock | 1 MHz (fixed) | 1.193182 MHz | CNTFRQ_EL0 (SoC-defined) | Platform-defined |
| Channels | 2 (Timer1 + Timer2) | 3 (ch0, ch1, ch2) | Per-CPU (physical + virtual) | Per-hart MTIMECMP |
| Mode | Free-running / periodic / one-shot | Interrupt on terminal count | Compare register TVAL/CVAL | MTIMECMP compare |
| Resolution | 1 µs (at 1 MHz) | 838 ns | Frequency-dependent | Frequency-dependent |
| UIOX base | 0x09010000 (ARM64) | Port 0x40 | System register MRS | 0x02000000 (CLINT) |

## 4.2  SP804 Register Map

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x000 | Timer1Load | R/W | Load register — value copied to counter when it reaches 0 (periodic mode) |
| 0x004 | Timer1Value | R | Current counter value (read-only, counts down) |
| 0x008 | Timer1Control | R/W | Control: EN[7]=enable, PERIODIC[6], IE[5]=IRQ enable, TIMSIZE[1]=32-bit, ONESHOT[0] |
| 0x00C | Timer1IntClr | W | Interrupt clear — write any value to clear pending interrupt |
| 0x010 | Timer1RIS | R | Raw interrupt status |
| 0x014 | Timer1MIS | R | Masked interrupt status (RIS AND Control.IE) |
| 0x018 | Timer1BGLoad | R/W | Background load — updated on next counter wrap (avoids glitch) |
| 0x020 | Timer2Load | R/W | Timer 2 load (same layout as Timer 1) |
| 0x028 | Timer2Control | R/W | Timer 2 control |
| 0x02C | Timer2IntClr | W | Timer 2 interrupt clear |
| 0xF00 | TimerITCR | R/W | Integration test control |

## 4.3  PIT 8254 Channels (x86)

| Port | Channel | Primary Use | UIOX Usage |
| --- | --- | --- | --- |
| 0x40 | Channel 0 | System clock tick (IRQ0) | 100 Hz periodic tick via Mode 3 (square wave) |
| 0x41 | Channel 1 | DRAM refresh (legacy) | Not used in UIOX |
| 0x42 | Channel 2 | PC speaker tone | Not used in UIOX |
| 0x43 | Command | Mode / byte select | Write 0x36 to configure ch0 rate generator lo/hi |

## 4.4  ARM Generic Timer — System Registers

| Register | Access | Description |
| --- | --- | --- |
| CNTFRQ_EL0 | MRS/MSR | Counter frequency in Hz (read at boot to calibrate delays) |
| CNTPCT_EL0 | MRS | Physical counter value (64-bit, monotonic) |
| CNTP_TVAL_EL0 | MRS/MSR | Physical timer value — time remaining to interrupt (signed) |
| CNTP_CTL_EL0 | MRS/MSR | Physical timer control: ENABLE[0], IMASK[1], ISTATUS[2] |
| CNTP_CVAL_EL0 | MRS/MSR | Physical timer comparator (absolute 64-bit value) |
| CNTVCT_EL0 | MRS | Virtual counter value |
| CNTV_TVAL_EL0 | MRS/MSR | Virtual timer value |
| CNTV_CTL_EL0 | MRS/MSR | Virtual timer control (same bits as physical) |

## 4.5  RISC-V CLINT Memory Map

| Address | Register | Description |
| --- | --- | --- |
| CLINT_BASE+0x0000 | MSIP[hart] | Machine software interrupt pending per hart (write 1 to trigger IPI) |
| CLINT_BASE+0x4000 | MTIMECMP[hart] | Machine timer compare register per hart (64-bit, fires MTIP when mtime >= MTIMECMP) |
| CLINT_BASE+0xBFF8 | MTIME | Machine time counter (64-bit, read-only in S-mode via rdtime CSR) |

# 5 · PCIe — PCI Express Host Controller

Header: 02_FwHal/include/uiox_fw_pcie.h

UIOX implements PCIe firmware initialisation covering ECAM config space access, BAR sizing and assignment, and command register enable. Devices supported: NVMe SSDs, SATA controllers, Ethernet NICs, and VGA/GPU.

## 5.1  PCIe Physical Layer Overview

| Layer | Description |
| --- | --- |
| Physical | Differential serial lanes (PCIe ×1/×2/×4/×8/×16). Each lane = 1 TX pair + 1 RX pair (4 wires) |
| Data Link | 128b/130b encoding (Gen 3+), ACK/NAK, flow control credits, Data Link Layer Packets (DLLP) |
| Transaction | TLP (Transaction Layer Packets): Memory Read/Write, I/O, Config, Message, Completion |
| Config Space | 4 KB per function, ECAM MMIO mapped: base + (bus<<20 ¦ dev<<15 ¦ fn<<12 ¦ offset) |

## 5.2  PCIe Signal Pins (×1 connector)

| Pin Group | Signals | Count | Description |
| --- | --- | --- | --- |
| TX differential | TX+, TX− | 2 | Transmit differential pair per lane |
| RX differential | RX+, RX− | 2 | Receive differential pair per lane |
| REFCLK | REFCLK+, REFCLK− | 2 | 100 MHz reference clock differential |
| PERST# | PERST# | 1 | PCIe reset, active low — held low at boot until stable |
| WAKE# | WAKE# | 1 | Wake from D3cold state |
| CLKREQ# | CLKREQ# | 1 | Clock request — device requests REFCLK |
| SMBUS | SMBCLK, SMBDAT | 2 | SMBus for out-of-band management |
| Power | 3.3V, 12V, GND | — | Power supply rails |

## 5.3  PCI Configuration Space Header (Type 0)

| Offset (hex) | Register | R/W | Bits | Description |
| --- | --- | --- | --- | --- |
| 00 | Vendor ID | R | 15:0 | PCI-SIG assigned vendor ID (0xFFFF = no device) |
| 02 | Device ID | R | 15:0 | Vendor-assigned device ID |
| 04 | Command | R/W | 15:0 | IO_EN[0], MEM_EN[1], BUS_MASTER[2], INT_DISABLE[10] |
| 06 | Status | R/WC | 15:0 | CAP_LIST[4], INT_STATUS[3], MASTER_DATA_PARITY_ERR[8] |
| 08 | Class Code / Rev | R | 31:0 | Rev ID[7:0], Prog IF[15:8], Sub-class[23:16], Class[31:24] |
| 0C | Cache Line Size | R/W | 7:0 | Cache line size in 32-bit words (for MWI transactions) |
| 0E | Header Type | R | 7:0 | Type[6:0]=0 endpoint, MF[7]=multifunction |
| 10 | BAR0 | R/W | 31:0 | Base Address Register 0 (MMIO or I/O space) |
| 14 | BAR1–BAR5 | R/W | — | Base Address Registers 1–5 (same format as BAR0) |
| 2C | Subsystem Vendor | R | 15:0 | Board-level vendor ID |
| 2E | Subsystem Device | R | 15:0 | Board-level device ID |
| 34 | Capability Pointer | R | 7:0 | Offset to first capability structure |
| 3C | Interrupt Line | R/W | 7:0 | IRQ line routed to (0xFF = no connection) |
| 3D | Interrupt Pin | R | 7:0 | INTA#=1, INTB#=2, INTC#=3, INTD#=4 |

## 5.4  ECAM Address Formula

PA = ecam_base + ((bus << 20) | (device << 15) | (function << 12) | register_offset)

ARM64 QEMU: ecam_base = 0x3F000000

x86-64 Q35: ecam_base = 0xB0000000

# 6 · USB — xHCI / EHCI / OHCI Host Controller

Header: 02_FwHal/include/uiox_fw_usb.h

## 6.1  USB Speed Tiers

| Speed | Enum | Rate | Connector | PHY |
| --- | --- | --- | --- | --- |
| Low Speed | UIOX_USB_SPEED_LS | 1.5 Mbit/s | USB-A/B | FS PHY |
| Full Speed | UIOX_USB_SPEED_FS | 12 Mbit/s | USB-A/B | FS PHY |
| High Speed | UIOX_USB_SPEED_HS | 480 Mbit/s | USB-A/B | HS PHY (EHCI) |
| SuperSpeed | UIOX_USB_SPEED_SS | 5 Gbit/s | USB-A/C | SS PHY (xHCI, USB 3.0) |
| SuperSpeed+ | UIOX_USB_SPEED_SS_PLUS | 10 Gbit/s | USB-C | SS+ PHY (xHCI, USB 3.1) |

## 6.2  USB Type-A / Type-C Physical Pins

| Pin | USB-A Name | USB-C Name | Description |
| --- | --- | --- | --- |
| 1 | VBUS | VBUS (A4,A9,B4,B9) | 5V power supply (500 mA–3A depending on negotiation) |
| 2 | D− | D− (A7,B7) | USB 2.0 differential data minus |
| 3 | D+ | D+ (A6,B6) | USB 2.0 differential data plus |
| 4 | GND | GND (A1,B1,A12,B12) | Ground |
| — | — | TX1+/TX1− (A2/A3) | USB 3.x SuperSpeed transmit lane 1 |
| — | — | RX1+/RX1− (B10/B11) | USB 3.x SuperSpeed receive lane 1 |
| — | — | TX2+/TX2− (B2/B3) | USB 3.x SuperSpeed transmit lane 2 (for alt modes) |
| — | — | RX2+/RX2− (A10/A11) | USB 3.x SuperSpeed receive lane 2 |
| — | — | CC1 (A5) | Configuration Channel 1 — cable orientation, power role |
| — | — | CC2 (B5) | Configuration Channel 2 |
| — | — | SBU1/SBU2 (A8/B8) | Sideband Use — audio/video in alt modes |

## 6.3  xHCI Key Registers

| Offset | Register | Description |
| --- | --- | --- |
| 0x00 | CAPLENGTH | Capability Length — offset to operational registers base |
| 0x04 | HCSPARAMS1 | Structural: MaxSlots[7:0], MaxIntrs[18:8], MaxPorts[31:24] |
| 0x08 | HCSPARAMS2 | Scratchpad buffer size |
| 0x10 | HCCPARAMS1 | Capability: 64-bit addressing, doorbell offset, context size |
| Op+0x00 | USBCMD | Run/Stop[0], HCRST[1]=host reset, EWE[3]=event wrap enable |
| Op+0x04 | USBSTS | HCH[0]=halted, HSE[2]=host system error, EINT[3]=event interrupt |
| Op+0x14 | DNCTRL | Device Notification Control |
| Op+0x18 | CRCR_LO | Command Ring Control Register low (physical address + RCS) |
| Op+0x30 | DCBAAP_LO | Device Context Base Address Array Pointer low |
| Op+0x38 | CONFIG | MaxSlotsEn[7:0] |

# 7 · SD / SDHC / SDXC / eMMC Host Controller

Header: 02_FwHal/include/uiox_fw_sd.h

UIOX supports SD/SDHC/SDXC cards via the BCM2835/QEMU SDIO host controller (SDIO 4-bit mode) or in SPI fallback mode through the PL022 SSP.

## 7.1  SD Card Physical Pins (SD Full-Size)

| Pin | SD Name | SPI Mode | Direction | Description |
| --- | --- | --- | --- | --- |
| 1 | CD/DAT3 | CS | In/Out | Card Detect / Data 3 / SPI Chip Select |
| 2 | CMD | DI | In/Out | Command / SPI Data In (MOSI) |
| 3 | VSS1 | VSS1 | — | Ground |
| 4 | VDD | VDD | — | Power supply 2.7–3.6V |
| 5 | CLK | SCLK | Input | Clock — up to 25 MHz (default speed) / 50 MHz (high speed) |
| 6 | VSS2 | VSS2 | — | Ground |
| 7 | DAT0 | DO | In/Out | Data 0 / SPI Data Out (MISO) |
| 8 | DAT1 | — | In/Out | Data 1 (unused in SPI mode) |
| 9 | DAT2 | — | In/Out | Data 2 (unused in SPI mode) |

## 7.2  SD Commands Used in UIOX

| Command | Code | Description |
| --- | --- | --- |
| CMD0 | SD_CMD0_IDLE=0 | GO_IDLE_STATE — reset card to idle state |
| CMD8 | SD_CMD8_IF_COND=8 | SEND_IF_COND — check operating voltage (SDHC/SDXC detection) |
| CMD17 | SD_CMD17_READ=17 | READ_SINGLE_BLOCK — read one 512-byte block at LBA |
| CMD24 | SD_CMD24_WRITE=24 | WRITE_BLOCK — write one 512-byte block |
| CMD55 | SD_CMD55_APP=55 | APP_CMD — prefix for ACMD commands |
| ACMD41 | SD_ACMD41_OP_COND=41 | SD_SEND_OP_COND — initiate card power-up; HCS[30]=1 for SDHC |

# 8 · NVMe — Non-Volatile Memory Express (M.2 PCIe)

Header: 02_FwHal/include/uiox_fw_nvme.h

NVMe devices connect via PCIe and are accessed through BAR0 MMIO registers. UIOX firmware sets up the Admin Queue and one I/O queue for block read/write before handing off to the kernel.

## 8.1  M.2 (NGFF) Physical Connector — Key M

| Pin Group | Pins | Signal | Description |
| --- | --- | --- | --- |
| PCIe lanes | 10,12 / 11,13 | TX0+/TX0− | PCIe ×4 transmit differential pairs (lanes 0–3) |
| PCIe lanes | 14,16 / 15,17 | RX0+/RX0− | PCIe ×4 receive differential pairs |
| Reference | 53,55 | REFCLK+/− | 100 MHz PCIe reference clock |
| Reset | 33 | PERST# | PCIe fundamental reset — held low at boot |
| Power | 2,3,71,72 | 3.3V | Main power rail |
| Ground | Various | GND | Signal ground |
| Sideband | 25,27 | SMBUS | SMBus for management |
| Wake | 32 | CLKREQ# | Clock request from device to host |

## 8.2  NVMe BAR0 Register Map

| Offset (hex) | Register | R/W | Description |
| --- | --- | --- | --- |
| 0000 | CAP | R | Controller Capabilities — MQES[15:0]=max queue entries, TO[31:24]=timeout, DSTRD[35:32]=doorbell stride |
| 0008 | VS | R | Version — MJR[31:16]=major, MNR[15:8]=minor, TER[7:0]=tertiary |
| 000C | INTMS | R/W | Interrupt Mask Set |
| 0010 | INTMC | R/W | Interrupt Mask Clear |
| 0014 | CC | R/W | Controller Configuration — EN[0]=enable, CSS[6:4]=cmd set, MPS[10:7]=page size, IOSQES[19:16], IOCQES[23:20] |
| 001C | CSTS | R | Controller Status — RDY[0]=ready, CFS[1]=fatal status, SHST[3:2]=shutdown status |
| 0024 | AQA | R/W | Admin Queue Attributes — ASQS[11:0]=sq size, ACQS[27:16]=cq size |
| 0028 | ASQ | R/W | Admin Submission Queue Base Address (64-bit physical) |
| 0030 | ACQ | R/W | Admin Completion Queue Base Address (64-bit physical) |
| 1000 | SQ0TDBL | R/W | Admin Submission Queue 0 Tail Doorbell — write new tail to submit commands |
| 1004 | CQ0HDBL | R/W | Admin Completion Queue 0 Head Doorbell — write new head to free entries |

# 9 · Network — Ethernet MAC/PHY (VirtIO-net / SMSC LAN9118)

Header: 02_FwHal/include/uiox_fw_net.h

## 9.1  RJ-45 Ethernet Physical Pins (10/100/1000BASE-T)

| Pin | 10/100BASE-T | 1000BASE-T | Description |
| --- | --- | --- | --- |
| 1 | TX+ | BI_DA+ | Transmit data + (10/100) / Bidirectional pair A+ |
| 2 | TX− | BI_DA− | Transmit data − |
| 3 | RX+ | BI_DB+ | Receive data + |
| 4 | — | BI_DC+ | Unused (10/100) / Bidirectional pair C+ |
| 5 | — | BI_DC− | Unused (10/100) / Bidirectional pair C− |
| 6 | RX− | BI_DB− | Receive data − |
| 7 | — | BI_DD+ | Unused (10/100) / Bidirectional pair D+ |
| 8 | — | BI_DD− | Unused (10/100) / Bidirectional pair D− |

## 9.2  SMSC LAN9118 Key Registers

| Offset | Register | Description |
| --- | --- | --- |
| 0x50 | TX_CFG | TX Enable[1], TX FIFO reset[2] |
| 0x54 | RX_CFG | RX FIFO alignment, DMA on/off |
| 0x58 | HW_CFG | TX FIFO size[18:16], MBO[20]=must be 1 |
| 0x64 | INT_STS | Interrupt status — clear on write |
| 0x68 | INT_EN | Interrupt enable mask |
| 0x6C | INT_CFG | IRQ polarity, INT_DEAS |
| 0xA4 | MAC_CSR | MAC control/status — Auto-negotiate, duplex, loopback |
| 0xA8 | MAC_DATA | Indirect MAC register data |
| 0xAC | MAC_CMD | Indirect MAC register command |

# 10 · Watchdog Timer — SP805 (ARM) / iTCO (x86)

Header: 02_FwHal/include/uiox_fw_wdt.h

The watchdog timer issues a hardware reset if the firmware fails to kick (reload) it within the configured timeout. UIOX firmware kicks the watchdog at each boot stage boundary.

## 10.1  SP805 Register Map

| Offset | Register | R/W | Description |
| --- | --- | --- | --- |
| 0x000 | WDOGLOAD | R/W | Load value — written to counter on kick; timeout = LOAD / WDOGCLK |
| 0x004 | WDOGVALUE | R | Current counter value (down-counter, read-only) |
| 0x008 | WDOGCONTROL | R/W | INTEN[0]=interrupt enable, RESEN[1]=reset enable |
| 0x00C | WDOGINTCLR | W | Interrupt clear / kick — write any value to reload counter |
| 0x010 | WDOGRIS | R | Raw interrupt status |
| 0x014 | WDOGMIS | R | Masked interrupt status |
| 0xC00 | WDOGLOCK | R/W | Write 0x1ACCE551 to unlock; write any other to lock register writes |

# 11 · PMIC — Power Management IC (DA9062 / RK808)

Header: 02_FwHal/include/uiox_fw_pmic.h

The PMIC manages all power rails on the SoC board via I2C. UIOX firmware programs BUCK converter voltages and LDO outputs during early boot to bring up the CPU and DDR at the correct voltages.

## 11.1  DA9062 I2C Interface

| Parameter | Value |
| --- | --- |
| I2C Address | 0x58 (7-bit, AD0=GND) or 0x59 (AD0=VDD) |
| I2C Speed | Standard (100 kHz) or Fast (400 kHz) |
| Bus | I2C0 or I2C1 (platform-specific) |
| IRQ | Active-low open-drain interrupt to SoC GPIO |
| Reset | nPMIC_RST active-low input from SoC |

## 11.2  DA9062 Key Registers

| Address (hex) | Register | Description |
| --- | --- | --- |
| 0x05 | STATUS_A | Fault status — TEMP_WARN, VBAT_OK, VDD_FAULT |
| 0x08 | FAULT_LOG | Latched fault log — cleared on read |
| 0x27 | SYSTEM_CTRL | POWER_EN[0], SHUTDOWN request |
| 0x9C | BUCK1_CFG | BUCK1 (VDD_CPU) voltage set — Vout = 300mV + N×12.5mV |
| 0x9D | BUCK1_ON | BUCK1 target voltage when ON |
| 0x9E | BUCK1_SLEEP | BUCK1 voltage in sleep mode |
| 0xA2 | LDO1_CFG | LDO1 voltage — 0.9V–3.6V in 50mV steps |
| 0xA3 | LDO2_CFG | LDO2 voltage |
| 0xD5 | CHIP_ID | Device ID — should read 0x62 for DA9062 |

## 11.3  Power Rail Assignments (typical SoC board)

| Rail | Source | Voltage | Load |
| --- | --- | --- | --- |
| VDD_CPU | BUCK1 | 0.8–1.05V | CPU cores (dynamic voltage scaling) |
| VDD_GPU | BUCK2 | 0.85V | GPU / media processors |
| VDD_DDR | BUCK3 | 1.1V / 1.35V / 1.8V | LPDDR4/DDR4 VDD |
| VDD_IO | LDO1 | 3.3V | GPIO, UART, SPI I/O ring |
| VDD_PHY | LDO2 | 1.8V | USB PHY, PCIe PHY, DDR PHY |
| VDD_RTC | LDO3 | 3.3V | RTC, crystal oscillator backup |
| VDD_WIFI | LDO4 | 3.3V | WiFi/BT module supply |

# 12 · Thermal Sensor & Trip-Point Management

Header: 02_FwHal/include/uiox_fw_thermal.h

UIOX monitors on-chip and external temperature sensors and takes protective action (clock throttling, fan control, shutdown) at configurable trip points.

## 12.1  Thermal Trip Points

| Trip Type | Typical Threshold | Action |
| --- | --- | --- |
| Passive | 75–85 °C | Reduce CPU/GPU frequency (DVFS) |
| Active | 70 °C | Turn on fan at low speed |
| Hot | 90 °C | Increase fan speed, warn kernel |
| Critical | 105 °C | Emergency shutdown — uiox_fw_power_reset() |

## 12.2  On-Chip Temperature Sensor (BCM2712 / typical ARM SoC)

| Register Offset | Name | Description |
| --- | --- | --- |
| 0x00 | TSENS_CTRL | ENABLE[0], AVG_NUM[3:1]=averaging count, RUNBIT[8] |
| 0x04 | TSENS_DATA | Raw ADC output (12-bit) — Temp_C = (raw − offset) / slope |
| 0x08 | TSENS_LIMIT | HIGH_LIMIT[11:0], LOW_LIMIT[27:16] for interrupt generation |
| 0x0C | TSENS_INT | OVER_LIMIT[0], UNDER_LIMIT[1] interrupt status |

# 13 · Sensor Interface (I2C / SPI Sensors)

Header: 02_FwHal/include/uiox_fw_sensor.h

UIOX provides a unified sensor abstraction for ambient light (TSL2591), IMU (BMI270), temperature, pressure, humidity, and proximity sensors connected via I2C or SPI.

## 13.1  Sensor Types and Devices

| Sensor Type | Enum | Typical Device | Bus | Address/CS |
| --- | --- | --- | --- | --- |
| Temperature | UIOX_FW_SENS_TEMP | TMP117 / NTC thermistor | I2C | 0x48 |
| Humidity | UIOX_FW_SENS_HUMIDITY | SHT31 | I2C | 0x44 |
| Pressure | UIOX_FW_SENS_PRESSURE | BMP388 | I2C/SPI | 0x76 / CS0 |
| Ambient Light | UIOX_FW_SENS_LIGHT | TSL2591 | I2C | 0x29 |
| IMU Accel | UIOX_FW_SENS_ACCEL | BMI270 | SPI | CS1 |
| IMU Gyro | UIOX_FW_SENS_GYRO | BMI270 | SPI | CS1 |
| Proximity | UIOX_FW_SENS_PROXIMITY | VCNL4040 | I2C | 0x60 |
| MMIO | UIOX_FW_SENS_MMIO | On-chip sensor | MMIO | Base addr |

## 13.2  I2C Signal Pins

| Pin | Name | Direction | Description |
| --- | --- | --- | --- |
| 1 | VDD | Power | Supply voltage 1.8V or 3.3V |
| 2 | GND | Ground | Signal and power ground |
| 3 | SDA | Bidirectional | Serial Data — open-drain, pulled up externally (4.7 kΩ typical) |
| 4 | SCL | Input | Serial Clock — open-drain, pulled up externally |
| 5 | INT | Output | Interrupt (optional, active low, open drain) |
| 6 | ADDR | Input | I2C address select (tied to VDD or GND to set LSB of address) |

# 14 · Complete Peripheral Register & Pin Summary

| Header File | Peripheral | IP Core | Bus | MMIO / Port | IRQ | Key Registers |
| --- | --- | --- | --- | --- | --- | --- |
| uiox_fw_uart.h | UART | PL011 / 16550A | APB / ISA | 0x09000000 | 33 | DR, FR, IBRD, FBRD, LCR_H, CR, IMSC, ICR |
| uiox_fw_spi.h | SPI Master | PL022 SSP | APB | 0x09050000 | — | CR0, CR1, DR, SR, CPSR, IMSC, ICR |
| uiox_fw_timer.h | Timer | SP804 / PIT / GT | APB / ISA | 0x09010000 | 27 | Load, Value, Control, IntClr |
| uiox_fw_pcie.h | PCIe | ECAM host | PCIe | 0x3F000000 | 32+ | Vendor/Device, BAR0-5, Command, Status |
| uiox_fw_usb.h | USB xHCI | xHCI 1.1 | PCIe BAR0 | BAR0 | MSI | CAPLENGTH, HCSPARAMS1, USBCMD, USBSTS, DCBAAP |
| uiox_fw_net.h | Ethernet | VirtIO / LAN9118 | VirtIO / AHB | VirtIO | IRQ | TX_CFG, RX_CFG, INT_STS, MAC_CSR |
| uiox_fw_sd.h | SD/SDIO | BCM SDIO / SDHCI | AXI | 0xFE340000 | 22 | CMD, ARG, RESP, DATA, STATUS |
| uiox_fw_nvme.h | NVMe | PCIe NVMe | PCIe BAR0 | BAR0 | MSI | CAP, CC, CSTS, AQA, ASQ, ACQ, SQ0TDBL |
| uiox_fw_wdt.h | Watchdog | SP805 / iTCO | APB | 0x09060000 | 32 | WDOGLOAD, WDOGCONTROL, WDOGINTCLR, WDOGLOCK |
| uiox_fw_pmic.h | PMIC | DA9062 / RK808 | I2C | I2C 0x58 | GPIO | BUCK1_CFG, LDO1_CFG, STATUS_A, CHIP_ID |
| uiox_fw_thermal.h | Thermal | On-chip / I2C | MMIO / I2C | 0x09070000 | — | TSENS_CTRL, TSENS_DATA, TSENS_LIMIT |
| uiox_fw_sensor.h | Sensors | TSL2591/BMI270/… | I2C / SPI | 0x29/CS1 | GPIO | Device-specific |
| uiox_fw_power.h | Power Mgmt | PSCI / ACPI / SBI | SMC / MMIO | SMC / 0xB004 | — | PSCI CPU_ON/OFF, SYSTEM_RESET, SYSTEM_OFF |
| uiox_fw_psci.h | PSCI | ARM PSCI 1.1 | SMC / HVC | SMC | — | 0x84000003 CPU_ON, 0x84000009 SYSTEM_RESET |
| uiox_fw_tz.h | TrustZone EL3 | ARM TZC-400 | AXI / EL3 | SMC / MMIO | — | SCR_EL3, VBAR_EL3, CPTR_EL3, TZC-400 regions |
| uiox_fw_secboot.h | Secure Boot | OTP / SHA-256 | OTP / MMIO | OTP | — | rot_hash, FVC, image_hdr, anti-rollback counter |
| uiox_fw_sata.h | SATA | AHCI | PCIe BAR5 | BAR5 | MSI | CAP, GHC, IS, PI, HBA Port registers |
| uiox_fw_storage.h | Storage Abstraction | VirtIO / eMMC | AXI / PCIe | Various | — | Unified block R/W API over SATA/NVMe/SD |
| uiox_fw_mem.h | DDR Init | DDR PHY / LPDDR4 | MMIO | PHY base | — | PHY training, ZQ calibration, MR registers |
| uiox_fw_ramrtc.h | RAM RTC | Software RTC | MMIO / I2C | 0x09080000 | — | epoch_sec, alarm_sec, ctrl |
| uiox_fw_wifi.h | WiFi | 43455/AW-CM256SM | SDIO / PCIe | SDIO | GPIO | Country, SSID, auth, scan results |
| uiox_fw_post.h | POST | Power-On Self Test | — | — | — | POST result codes, test sequence |
| uiox_fw_monitor.h | Display | HDMI / DSI | MMIO | 0x3F90D000 | — | Resolution, pixel format, framebuffer addr |
| uiox_fw_usb.h (TB4) | Thunderbolt 4 | TBT4 controller | PCIe | PCIe BAR | MSI | TUSB controller regs, alt-mode negotiation |

UIOX 02_FwHal Hardware Reference  ·  Generated 2026-07-14  ·  github.com/Pramod645/UIOX