UIOX Project · Instruction Set Reference

# ISA Reference Manual

ARM32 · ARM64 · x86-64 · RISC-V  —  Architecture, Instruction Sets & Integration Guide

# 1 · Overview

The UIOX kernel targets four processor families: ARMv7-A (arm32), ARMv8-A/AArch64 (arm64), AMD64/Intel 64 (x86-64), and RISC-V (RV32I/RV64I). Each architecture is implemented under 10_Arch/ with dedicated arch_init.c, arch_defs.h, memory, exception, and decode layers.

This document provides a reference for the instruction set of each architecture as used within the UIOX codebase — covering general-purpose instructions, system/privileged instructions, inline-assembly patterns, and UIOX-specific integration notes for the security BSP, memory management, and interrupt handling.

| Architecture | Variant | UIOX Module | Word Size | Endian |
| --- | --- | --- | --- | --- |
| ARM32 | ARMv7-A | 10_Arch/arm32 | 32-bit | Little |
| ARM64 | ARMv8-A / AArch64 | 10_Arch/arm64 | 64-bit | Little |
| x86-64 | AMD64 / Intel 64 | 10_Arch/x86_64 | 64-bit | Little |
| RISC-V | RV32I / RV64I + M/A/F/D | 10_Arch/riscv | 32/64-bit | Little |

# 2 · ARM32 (ARMv7-A)

## 2.1  Register File

ARMv7-A provides 16 general-purpose 32-bit registers visible at any one time, numbered R0–R15. The architecture defines several banked register sets that are swapped in automatically on exception entry.

| Register | ABI Name | Role |
| --- | --- | --- |
| R0–R3 | a1–a4 | Argument / return value registers |
| R4–R11 | v1–v8 | Callee-saved variable registers |
| R11 | fp | Frame pointer (optional, by convention) |
| R12 | ip | Intra-procedure scratch (linker veneer) |
| R13 | sp | Stack pointer |
| R14 | lr | Link register — return address from BL/BLX |
| R15 | pc | Program counter (readable/writable) |
| CPSR | — | Current Program Status Register (flags + mode) |
| SPSR | — | Saved PSR — holds pre-exception CPSR |

## 2.2  Processor Modes

ARMv7-A implements eight processor modes. UIOX runs the kernel in Supervisor (SVC) mode and user tasks in User mode. FIQ, IRQ, Abort, and Undefined modes are used by the exception vector table installed via VBAR.

| Mode | Encoding | Purpose |
| --- | --- | --- |
| User | 0b10000 | Unprivileged task execution |
| FIQ | 0b10001 | Fast interrupt — has R8–R14 banked |
| IRQ | 0b10010 | Normal interrupt |
| Supervisor | 0b10011 | Kernel / SVC call handler (UIOX default) |
| Monitor | 0b10110 | TrustZone Secure Monitor (SMC) |
| Abort | 0b10111 | Prefetch / data abort handler |
| Undefined | 0b11011 | Undefined instruction handler |
| System | 0b11111 | Privileged, shares User register bank |

## 2.3  Core Instruction Set

### 2.3.1  Data Processing

All data-processing instructions follow the format: <opcode>{S}{cond} Rd, Rn, operand2. The S suffix updates CPSR flags; cond is a 4-bit condition code (EQ, NE, GT, LT, …).

| Instruction | Syntax | Operation | Flags |
| --- | --- | --- | --- |
| MOV | MOV Rd, op2 | Rd ← op2 | N,Z,C |
| MVN | MVN Rd, op2 | Rd ← ~op2 | N,Z,C |
| ADD | ADD Rd, Rn, op2 | Rd ← Rn + op2 | N,Z,C,V |
| ADC | ADC Rd, Rn, op2 | Rd ← Rn + op2 + C | N,Z,C,V |
| SUB | SUB Rd, Rn, op2 | Rd ← Rn − op2 | N,Z,C,V |
| SBC | SBC Rd, Rn, op2 | Rd ← Rn − op2 − ~C | N,Z,C,V |
| RSB | RSB Rd, Rn, op2 | Rd ← op2 − Rn | N,Z,C,V |
| AND | AND Rd, Rn, op2 | Rd ← Rn & op2 | N,Z,C |
| ORR | ORR Rd, Rn, op2 | Rd ← Rn ¦ op2 | N,Z,C |
| EOR | EOR Rd, Rn, op2 | Rd ← Rn ^ op2 | N,Z,C |
| BIC | BIC Rd, Rn, op2 | Rd ← Rn & ~op2 | N,Z,C |
| CMP | CMP Rn, op2 | Set flags on Rn − op2 (no result) | N,Z,C,V |
| TST | TST Rn, op2 | Set flags on Rn & op2 | N,Z,C |
| MUL | MUL Rd, Rm, Rs | Rd ← Rm × Rs (low 32) | N,Z |
| MLA | MLA Rd, Rm, Rs, Rn | Rd ← Rm × Rs + Rn | N,Z |
| UMULL | UMULL RdLo,RdHi,Rm,Rs | RdHi:RdLo ← Rm × Rs (64-bit) | N,Z |

### 2.3.2  Shift / Rotate (Operand2 modifiers)

| Modifier | Meaning |
| --- | --- |
| LSL #n / LSL Rx | Logical shift left by n bits or contents of Rx |
| LSR #n / LSR Rx | Logical shift right (zero fill) |
| ASR #n / ASR Rx | Arithmetic shift right (sign extend) |
| ROR #n / ROR Rx | Rotate right |
| RRX | Rotate right one bit through carry |

### 2.3.3  Load / Store

| Instruction | Syntax | Operation |
| --- | --- | --- |
| LDR | LDR Rt, [Rn, #off] | Rt ← Mem32[Rn+off] |
| STR | STR Rt, [Rn, #off] | Mem32[Rn+off] ← Rt |
| LDRB | LDRB Rt, [Rn, #off] | Rt ← zero-extend(Mem8[Rn+off]) |
| STRB | STRB Rt, [Rn, #off] | Mem8[Rn+off] ← Rt[7:0] |
| LDRH | LDRH Rt, [Rn, #off] | Rt ← zero-extend(Mem16[Rn+off]) |
| STRH | STRH Rt, [Rn, #off] | Mem16[Rn+off] ← Rt[15:0] |
| LDRD | LDRD Rt, Rt2, [Rn, #off] | Rt:Rt2 ← Mem64[Rn+off] |
| LDM | LDM Rn, {reglist} | Load multiple registers from [Rn] |
| STM | STM Rn, {reglist} | Store multiple registers to [Rn] |
| PUSH | PUSH {reglist} | STM SP!, {reglist} — push to stack |
| POP | POP {reglist} | LDM SP!, {reglist} — pop from stack |

### 2.3.4  Branch

| Instruction | Operation |
| --- | --- |
| B label | PC ← label (±32 MB range) |
| BL label | LR ← PC+4; PC ← label (subroutine call) |
| BX Rm | PC ← Rm (bit[0]=1 → switch to Thumb) |
| BLX Rm | LR ← PC+4; PC ← Rm (call + possible mode switch) |
| BLX label | Unconditional BL + Thumb/ARM switch |
| CBZ Rn, lbl | Branch if Rn == 0 (Thumb-2 only) |
| CBNZ Rn,lbl | Branch if Rn != 0 (Thumb-2 only) |

### 2.3.5  System / Privileged

| Instruction | Operation |
| --- | --- |
| SWI / SVC #imm | Supervisor Call — triggers SVC exception → UIOX syscall entry |
| SMC #imm | Secure Monitor Call — enters Monitor mode (TrustZone SMCCC) |
| MRS Rd, <sysreg> | Read special register (CPSR, SPSR, CP15 regs) into Rd |
| MSR <sysreg>, Rn | Write Rn to special register |
| MRC p15,op1,Rt,CRn,CRm,op2 | Read CP15 coprocessor register |
| MCR p15,op1,Rt,CRn,CRm,op2 | Write CP15 coprocessor register |
| MRRC p15,op,Rt,Rt2,CRm | Read 64-bit CP15 register (e.g. CNTPCT) |
| ISB | Instruction Synchronisation Barrier |
| DSB sy | Data Synchronisation Barrier — full system |
| DMB sy | Data Memory Barrier — ordering only |
| CLREX | Clear exclusive monitor (after LDREX/STREX failure) |
| LDREX Rt, [Rn] | Load-Exclusive 32-bit (begin atomic RMW) |
| STREX Rd, Rt,[Rn] | Store-Exclusive — Rd=0 if succeeded |
| WFI | Wait For Interrupt (low-power idle) |
| WFE | Wait For Event |
| SEV | Send Event (wake WFE on other core) |

## 2.4  UIOX ARM32 Inline Assembly Patterns

GIC IRQ enable (CPSR I-bit clear):

__asm__ volatile("cpsie i" ::: "memory");

Read cycle counter (PMCCNTR):

__asm__ volatile("mrc p15, 0, %0, c9, c13, 0" : "=r"(val));

ARMv7 generic timer (CNTPCT 64-bit read):

__asm__ volatile("mrrc p15, 0, %0, %1, c14" : "=r"(lo), "=r"(hi));

SP804 timer MMIO barrier after write:

__asm__ volatile("dsb sy" ::: "memory");

# 3 · ARM64 (ARMv8-A / AArch64)

## 3.1  Register File

AArch64 provides 31 general-purpose 64-bit registers (X0–X30) plus XZR (zero register) and SP. The lower 32 bits of each register are accessed as W0–W30. There is no visible PC in the register file; PC is only accessible via branch instructions and system reads.

| Register | ABI Name | Role |
| --- | --- | --- |
| X0–X7 | arg/ret | Arguments and return values (first 8) |
| X8 | indirect | Indirect result / syscall number (Linux ABI) |
| X9–X15 | scratch | Caller-saved temporaries |
| X16–X17 | IP0–IP1 | Intra-procedure scratch (linker / PLT) |
| X18 | platform | Platform register (reserved on some ABIs) |
| X19–X28 | callee | Callee-saved registers |
| X29 | fp | Frame pointer |
| X30 | lr | Link register (return address from BL) |
| SP | sp | Stack pointer (16-byte aligned in ABI) |
| XZR/WZR | — | Reads as 0; writes discarded |
| NZCV | — | Condition flags (N,Z,C,V) |
| DAIF | — | IRQ/FIQ/SError/Debug masks |
| ELR_ELn | — | Exception Link Register (per EL) |
| SPSR_ELn | — | Saved PSTATE (per EL) |

## 3.2  Exception Levels

| Level | Name | UIOX Role |
| --- | --- | --- |
| EL0 | User | Unprivileged process / task execution |
| EL1 | Kernel | UIOX kernel — interrupt handlers, MMU, scheduler |
| EL2 | Hypervisor | Optional VM host (not used in base UIOX) |
| EL3 | Secure Monitor | TrustZone firmware — SMC calls from UIOX BSP |

## 3.3  Core Instruction Set

### 3.3.1  Data Processing

| Instruction | Syntax | Operation |
| --- | --- | --- |
| MOV | MOV Xd, Xn / #imm | Xd ← Xn or immediate |
| MOVZ | MOVZ Xd, #imm{, LSL #n} | Xd ← zero-extended imm shifted |
| MOVK | MOVK Xd, #imm, LSL #n | Insert 16-bit immediate into Xd, keep others |
| MOVN | MOVN Xd, #imm{, LSL #n} | Xd ← ~(imm shifted) |
| ADD | ADD Xd, Xn, Xm{, shift} | Xd ← Xn + Xm |
| ADDS | ADDS Xd, Xn, Xm | ADD and set flags |
| ADC | ADC Xd, Xn, Xm | Xd ← Xn + Xm + C |
| SUB | SUB Xd, Xn, Xm{, shift} | Xd ← Xn − Xm |
| SUBS | SUBS Xd, Xn, Xm | SUB and set flags (also CMP alias) |
| NEG | NEG Xd, Xm | Xd ← 0 − Xm |
| MUL | MUL Xd, Xn, Xm | Xd ← Xn × Xm (low 64 bits) |
| UMULH | UMULH Xd, Xn, Xm | Xd ← high 64 bits of Xn × Xm (unsigned) |
| SMULH | SMULH Xd, Xn, Xm | Xd ← high 64 bits of Xn × Xm (signed) |
| UDIV | UDIV Xd, Xn, Xm | Xd ← Xn / Xm (unsigned integer) |
| SDIV | SDIV Xd, Xn, Xm | Xd ← Xn / Xm (signed integer) |
| AND | AND Xd, Xn, #imm/Xm | Xd ← Xn & operand |
| ORR | ORR Xd, Xn, #imm/Xm | Xd ← Xn ¦ operand |
| EOR | EOR Xd, Xn, #imm/Xm | Xd ← Xn ^ operand |
| BIC | BIC Xd, Xn, Xm | Xd ← Xn & ~Xm |
| LSL | LSL Xd, Xn, #n/Xm | Xd ← Xn << n |
| LSR | LSR Xd, Xn, #n/Xm | Xd ← Xn >> n (logical) |
| ASR | ASR Xd, Xn, #n/Xm | Xd ← Xn >> n (arithmetic) |
| ROR | ROR Xd, Xn, #n/Xm | Xd ← rotate right |
| CMP | CMP Xn, Xm/#imm | Set flags on Xn − operand (alias SUBS XZR,…) |
| CMN | CMN Xn, Xm/#imm | Set flags on Xn + operand |
| TST | TST Xn, Xm/#imm | Set flags on Xn & operand |
| CLZ | CLZ Xd, Xn | Count leading zeros |
| RBIT | RBIT Xd, Xn | Reverse bit order |
| REV | REV Xd, Xn | Reverse byte order (64-bit) |

### 3.3.2  Load / Store

| Instruction | Syntax | Operation |
| --- | --- | --- |
| LDR | LDR Xt, [Xn, #off] | Xt ← Mem64[Xn+off] |
| STR | STR Xt, [Xn, #off] | Mem64[Xn+off] ← Xt |
| LDR (W) | LDR Wt, [Xn, #off] | Wt ← Mem32[Xn+off] (zero-extend) |
| LDRB | LDRB Wt, [Xn, #off] | Wt ← Mem8[Xn+off] (zero-extend) |
| LDRH | LDRH Wt, [Xn, #off] | Wt ← Mem16[Xn+off] (zero-extend) |
| LDRSB | LDRSB Xt, [Xn, #off] | Xt ← sign-extend(Mem8[Xn+off]) |
| LDRSH | LDRSH Xt, [Xn, #off] | Xt ← sign-extend(Mem16[Xn+off]) |
| LDRSW | LDRSW Xt, [Xn, #off] | Xt ← sign-extend(Mem32[Xn+off]) |
| LDP | LDP Xt1,Xt2,[Xn,#off] | Load pair of 64-bit registers |
| STP | STP Xt1,Xt2,[Xn,#off] | Store pair of 64-bit registers |
| LDXR | LDXR Xt, [Xn] | Load-Exclusive (begin atomic RMW) |
| STXR | STXR Ws, Xt, [Xn] | Store-Exclusive; Ws=0 if success |
| LDADD | LDADD Xs, Xt, [Xn] | Atomic add (ARMv8.1-A LSE) |
| LDAR | LDAR Xt, [Xn] | Load-Acquire (all later accesses ordered after) |
| STLR | STLR Xt, [Xn] | Store-Release (all earlier accesses ordered before) |

### 3.3.3  Branch

| Instruction | Operation |
| --- | --- |
| B label | PC ← label (±128 MB) |
| BL label | X30 ← PC+4; PC ← label |
| BR Xn | PC ← Xn (indirect branch) |
| BLR Xn | X30 ← PC+4; PC ← Xn |
| RET | PC ← X30 (return) |
| RET Xn | PC ← Xn (return via alternate LR) |
| CBZ Xn,lbl | Branch if Xn == 0 |
| CBNZ Xn,lbl | Branch if Xn != 0 |
| TBZ Xn,#b,l | Branch if bit b of Xn == 0 |
| TBNZ Xn,#b,l | Branch if bit b of Xn != 0 |
| B.cond label | Conditional branch (EQ/NE/LT/GT/LE/GE/CS/CC/MI/PL/VS/VC/HI/LS) |

### 3.3.4  System / Privileged

| Instruction | Operation |
| --- | --- |
| SVC #imm | Supervisor Call → EL1 exception (UIOX syscall entry) |
| SMC #imm | Secure Monitor Call → EL3 (SMCCC TRNG, power mgmt) |
| HVC #imm | Hypervisor Call → EL2 |
| ERET | Exception Return — restores PC←ELR_ELn, PSTATE←SPSR_ELn |
| MSR <sysreg>, Xt | Write system register (see table 3.3.5) |
| MRS Xt, <sysreg> | Read system register |
| ISB | Instruction Synchronisation Barrier |
| DSB sy/st/ld/… | Data Synchronisation Barrier |
| DMB sy/st/ld/ish/… | Data Memory Barrier (ordering only) |
| DC CIVAC, Xt | Data cache clean+invalidate by VA to PoC |
| DC CVAC, Xt | Data cache clean by VA to PoC |
| IC IALLU | Instruction cache invalidate all to PoU |
| TLBI VMALLE1 | TLB invalidate all EL1 |
| AT S1E1R, Xt | Address translate stage 1 EL1 read |
| WFI | Wait For Interrupt (idle / low-power) |
| WFE | Wait For Event |
| SEV / SEVL | Send Event / Send Event Local |
| MRS Xt, RNDR | Read hardware TRNG (ARMv8.5-A FEAT_RNG) |
| MRS Xt, RNDRS | Read TRNG with reseeding |
| MRS Xt, CNTPCT_EL0 | Read physical counter (uiox_sec_plat_time_ms) |
| MRS Xt, CNTFRQ_EL0 | Read counter frequency in Hz |
| MRS Xt, ID_AA64ISAR0_EL1 | Feature register: RNDR support in bits[63:60] |

### 3.3.5  Key System Registers Used in UIOX

| Register | Purpose | UIOX Usage |
| --- | --- | --- |
| SCTLR_EL1 | System Control: MMU enable, cache, alignment | arch_init: MMU on |
| TTBR0_EL1 | Translation Table Base Register 0 (user) | MM: per-process page table |
| TTBR1_EL1 | Translation Table Base Register 1 (kernel) | MM: kernel page table |
| TCR_EL1 | Translation Control Register | MM: VA range, granule, shareability |
| MAIR_EL1 | Memory Attribute Indirection Register | MM: attribute index mapping |
| VBAR_EL1 | Vector Base Address Register | Exception: vector table base |
| ESR_EL1 | Exception Syndrome Register | Exception: syndrome decode |
| FAR_EL1 | Fault Address Register | Exception: faulting VA |
| ELR_EL1 | Exception Link Register | Exception: return PC after ERET |
| SPSR_EL1 | Saved Program Status | Exception: return PSTATE |
| SP_EL0 | User-mode stack pointer | Proc: task stack |
| TPIDR_EL0 | Thread Pointer (user) | Proc: TLS base |
| DAIF | IRQ/FIQ/SError/Debug masks | IRQ enable/disable |
| ICC_SRE_EL1 | GIC system register enable | GIC-v3 init |
| ICC_CTLR_EL1 | GIC CPU interface control | GIC-v3 priority |

## 3.4  UIOX ARM64 Inline Assembly Patterns

IRQ disable / enable via DAIF:

__asm__ volatile("msr daifset, #2" ::: "memory");  // disable IRQ

__asm__ volatile("msr daifclr, #2" ::: "memory");  // enable  IRQ

Read RNDR with carry-check (uiox_sec_plat_arm64.c):

__asm__ volatile("mrs %0, rndr  \n\t" "cset %w1, cs" : "=r"(val), "=r"(ok));

SMCCC SMC64 call (TRNG_RND64, PSCI):

__asm__ volatile("smc #0" : "+r"(x0),"+r"(x1),"+r"(x2),"+r"(x3));

TLB invalidate after page table update:

__asm__ volatile("dsb ishst \n\t" "tlbi vmalle1is \n\t" "dsb ish \n\t" "isb");

Atomic compare-and-swap (ARMv8.1 LSE):

__asm__ volatile("cas %0, %2, [%3]" : "+r"(old),"=r"(result) : "r"(new),"r"(addr));

# 4 · x86-64 (AMD64 / Intel 64)

## 4.1  Register File

x86-64 extends the x86 register set to 16 general-purpose 64-bit registers (RAX–R15), plus 16 SSE/AVX XMM/YMM/ZMM registers for SIMD. UIOX uses the System V AMD64 ABI on this target.

| Register | ABI Role | Notes |
| --- | --- | --- |
| RAX | Return value / syscall return | Also EAX (32-bit), AX (16-bit), AL/AH (8-bit) |
| RBX | Callee-saved | Base register (historical) |
| RCX | 4th arg / loop counter | Caller-saved |
| RDX | 3rd arg / I/O, RDTSC high | Caller-saved |
| RSI | 2nd arg | Caller-saved |
| RDI | 1st arg | Caller-saved |
| RSP | Stack pointer | 16-byte aligned at call boundary |
| RBP | Frame pointer (optional) | Callee-saved |
| R8–R9 | 5th–6th arg | Caller-saved |
| R10–R11 | Scratch | Caller-saved; syscall link |
| R12–R15 | Callee-saved | Preserved across calls |
| RIP | Instruction pointer | Read via LEA RIP-relative; not directly writable |
| RFLAGS | Status / control flags | CF, PF, AF, ZF, SF, OF, IF, DF |
| CS/DS/ES/FS/GS/SS | Segment registers | FS/GS used for thread-local storage in UIOX |

## 4.2  Privilege Levels (Rings)

| Ring | Purpose | UIOX Usage |
| --- | --- | --- |
| Ring 0 | Kernel mode — full hardware access | UIOX kernel, arch_init, interrupt handlers |
| Ring 1 | Not used by UIOX | — |
| Ring 2 | Not used by UIOX | — |
| Ring 3 | User mode — restricted access | User-space tasks, shell (50_UIX) |

## 4.3  Core Instruction Set

### 4.3.1  Data Transfer

| Instruction | Syntax | Operation |
| --- | --- | --- |
| MOV | MOV dst, src | dst ← src (register, memory, or immediate) |
| MOVZX | MOVZX dst, src | dst ← zero-extend(src) |
| MOVSX | MOVSX dst, src | dst ← sign-extend(src) |
| MOVSXD | MOVSXD Rq, Rd | Rq ← sign-extend(Rd) — 32→64 bit |
| XCHG | XCHG op1, op2 | Atomic swap (implicit LOCK with memory operand) |
| CMPXCHG | CMPXCHG dst, src | if RAX==dst then dst←src; else RAX←dst |
| XADD | XADD dst, src | tmp←dst+src; dst←src; src←tmp (atomic with LOCK) |
| LEA | LEA Rq, [addr] | Rq ← effective address (no memory access) |
| PUSH | PUSH src | RSP-=8; [RSP]←src |
| POP | POP dst | dst←[RSP]; RSP+=8 |
| CMOVCC | CMOV<cc> dst, src | Conditional move if condition code <cc> is true |

### 4.3.2  Arithmetic & Logic

| Instruction | Syntax | Operation | Flags |
| --- | --- | --- | --- |
| ADD | ADD dst, src | dst ← dst + src | CF,OF,SF,ZF,AF,PF |
| ADC | ADC dst, src | dst ← dst + src + CF | CF,OF,SF,ZF,AF,PF |
| SUB | SUB dst, src | dst ← dst − src | CF,OF,SF,ZF,AF,PF |
| SBB | SBB dst, src | dst ← dst − src − CF | CF,OF,SF,ZF,AF,PF |
| IMUL | IMUL Rq, Rq, imm | dst ← src1 × imm (signed) | CF,OF |
| MUL | MUL src | RDX:RAX ← RAX × src (unsigned) | CF,OF |
| IDIV | IDIV src | RAX ← RDX:RAX / src (signed); RDX ← remainder | undefined |
| INC | INC dst | dst ← dst + 1 | OF,SF,ZF,AF,PF (not CF) |
| DEC | DEC dst | dst ← dst − 1 | OF,SF,ZF,AF,PF (not CF) |
| NEG | NEG dst | dst ← 0 − dst | CF,OF,SF,ZF,AF,PF |
| AND | AND dst, src | dst ← dst & src | SF,ZF,PF; CF=OF=0 |
| OR | OR dst, src | dst ← dst ¦ src | SF,ZF,PF; CF=OF=0 |
| XOR | XOR dst, src | dst ← dst ^ src | SF,ZF,PF; CF=OF=0 |
| NOT | NOT dst | dst ← ~dst | — |
| SHL | SHL dst, CL/#n | Logical/arith shift left | CF,OF,SF,ZF,PF |
| SHR | SHR dst, CL/#n | Logical shift right | CF,OF,SF,ZF,PF |
| SAR | SAR dst, CL/#n | Arithmetic shift right | CF,OF,SF,ZF,PF |
| ROL | ROL dst, CL/#n | Rotate left | CF,OF |
| ROR | ROR dst, CL/#n | Rotate right | CF,OF |
| BT | BT src, bit | CF ← bit[n] of src | CF |
| BSF | BSF dst, src | Bit scan forward (index of LSB) | ZF |
| BSR | BSR dst, src | Bit scan reverse (index of MSB) | ZF |
| POPCNT | POPCNT dst, src | Count set bits | ZF |
| LZCNT | LZCNT dst, src | Count leading zeros | CF,ZF |
| TZCNT | TZCNT dst, src | Count trailing zeros | CF,ZF |
| CMP | CMP op1, op2 | Set flags on op1−op2 (no result) | CF,OF,SF,ZF,AF,PF |
| TEST | TEST op1, op2 | Set flags on op1&op2 (no result) | SF,ZF,PF; CF=OF=0 |

### 4.3.3  Control Flow

| Instruction | Operation |
| --- | --- |
| JMP rel/abs | Unconditional jump (near ±2 GB or far) |
| CALL rel/abs | Push RIP; jump (function call) |
| RET / RET n | Pop RIP; return (optionally add n to RSP) |
| Jcc label | Jump if condition: JE/JZ, JNE/JNZ, JL/JNGE, JG/JNLE, JLE, JGE, JB/JNAE, JA/JNBE, JS, JNS, JO, JNO, JP, JNP |
| LOOP label | RCX--; jump if RCX != 0 |
| SYSCALL | User→Ring 0 fast system call; RCX←RIP, R11←RFLAGS |
| SYSRET | Return from SYSCALL; RIP←RCX, RFLAGS←R11 |
| INT n | Software interrupt (legacy; INT 0x80 = Linux compat syscall) |
| IRETQ | Return from interrupt (64-bit); pops RIP, CS, RFLAGS, RSP, SS |

### 4.3.4  Memory Ordering & Atomics

| Instruction / Prefix | Operation |
| --- | --- |
| MFENCE | Full memory fence — all loads and stores ordered |
| SFENCE | Store fence — all stores ordered (SSE stores) |
| LFENCE | Load fence — all loads ordered; also serialises RDTSC/RDRAND |
| LOCK prefix | Prefix for RMW: LOCK CMPXCHG, LOCK XADD, LOCK INC etc. — full bus lock |
| PAUSE | Spin-wait hint — avoids memory-order machine clear in spin loops |
| CLFLUSH addr | Flush cache line containing addr to memory |
| CLWB addr | Clean (write-back) cache line without invalidating |

### 4.3.5  System / Privileged

| Instruction | Operation |
| --- | --- |
| RDTSC | Read timestamp counter → EDX:EAX (uiox_sec_plat_time_ms) |
| RDTSCP | RDTSC + read IA32_TSC_AUX into ECX (serialising) |
| RDRAND Rq | Read hardware CSPRNG (AES-CTR DRBG) → Rq; CF=1 if valid |
| RDSEED Rq | Read raw conditioned entropy → Rq; CF=1 if valid |
| CPUID | CPU identification/feature query (EAX=leaf, ECX=subleaf → EAX/EBX/ECX/EDX) |
| WRMSR | Write MSR[ECX] ← EDX:EAX (Ring 0) |
| RDMSR | Read MSR[ECX] → EDX:EAX (Ring 0) |
| LGDT | Load GDT register |
| LIDT | Load IDT register |
| LTR | Load task register (TSS) |
| MOV CR0–CR4 | Read/write control registers (Ring 0) |
| MOV DR0–DR7 | Read/write debug registers |
| INVLPG addr | Invalidate TLB entry for addr |
| INVPCID | Invalidate TLB by PCID |
| SWAPGS | Swap GS.base with IA32_KERNEL_GS_BASE (syscall entry/exit) |
| CLI / STI | Clear / set interrupt flag (Ring 0) |
| HLT | Halt until next interrupt (Ring 0 only) |
| IN / OUT | Port I/O read / write (Ring 0 or IOPL) |

## 4.4  CPUID Feature Leaves Used in UIOX

| EAX (leaf) | Bit | Feature | UIOX usage |
| --- | --- | --- | --- |
| 0x01 | ECX[30] | RDRAND | uiox_sec_plat_x86_64.c — entropy fallback |
| 0x07 | EBX[18] | RDSEED | uiox_sec_plat_x86_64.c — primary entropy |
| 0x07 | EBX[0] | FSGSBASE | FS/GS base read/write without MSR |
| 0x01 | EDX[25] | SSE | arch: XMM save/restore on context switch |
| 0x01 | ECX[28] | AVX | arch: YMM save/restore (XSAVE) |
| 0x07 | EBX[5] | AVX2 | Optional SIMD acceleration |
| 0x07 | ECX[2] | UMIP | User-Mode Instruction Prevention — restrict SGDT/SIDT |
| 0x07 | ECX[5] | WAITPKG | TPAUSE/UMONITOR — user-space sleep (future) |

## 4.5  UIOX x86-64 Inline Assembly Patterns

RDRAND with retry (uiox_sec_plat_x86_64.c):

__asm__ volatile("rdrand %0 \n\t" "setc %1" : "=r"(val), "=qm"(cf));

RDSEED with PAUSE between retries:

__asm__ volatile("rdseed %0 \n\t" "setc %1" : "=r"(val), "=qm"(cf));

__asm__ volatile("pause" ::: "memory");

RDTSC timestamp:

__asm__ volatile("rdtsc" : "=a"(lo), "=d"(hi));

CPUID leaf 0x07 (RDSEED detection):

__asm__ volatile("cpuid" : "=a"(*a),"=b"(*b),"=c"(*c),"=d"(*d) : "a"(7u),"c"(0u));

Atomic compare-and-swap (LOCK CMPXCHG):

__asm__ volatile("lock; cmpxchgq %2,%1" : "=a"(old),"+m"(*ptr) : "r"(new),"0"(old) : "cc");

SWAPGS at syscall entry:

__asm__ volatile("swapgs" ::: "memory");

# 5 · RISC-V (RV32I / RV64I + Extensions)

## 5.1  Design Philosophy

RISC-V is an open, royalty-free ISA designed for clean-slate simplicity. It is a fixed-width (32-bit instructions in the base ISA) load-store architecture with a minimal mandatory base (RV32I / RV64I) and optional standard extensions selected at build time. UIOX targets RV64IMAFDC — the 64-bit integer base plus Multiply, Atomic, single-precision Float, Double-precision float, and Compressed (16-bit) instructions.

| Extension | Letter | Description |
| --- | --- | --- |
| Integer Base | I | 32 or 64 registers, basic ALU, load/store, branches |
| Multiply/Divide | M | MUL, MULH, DIV, REM and their unsigned/word variants |
| Atomics | A | LR/SC (load-reserved/store-conditional), AMO instructions |
| Single-Prec Float | F | 32-bit IEEE 754 float registers f0–f31, FP ALU |
| Double-Prec Float | D | 64-bit IEEE 754 double (extends F) |
| Compressed | C | 16-bit encodings of common instructions (code density) |
| Bit Manipulation | B | CLZ, CTZ, CPOP, ANDN, ORN, XNOR, … |
| Cryptography | K | AES, SHA-2, SHA-3 scalar acceleration |
| Vector | V | Variable-length SIMD (RVV 1.0) |
| Zicsr | Zicsr | CSR read/write instructions (CSRRW, CSRRS, …) |
| Zifencei | Zifencei | FENCE.I instruction for I-cache coherence |

## 5.2  Register File

RISC-V defines 32 general-purpose integer registers (x0–x31) and 32 floating-point registers (f0–f31). Register x0 is hardwired to zero. The RISC-V calling convention assigns ABI names to all registers.

| Register | ABI Name | Role | Saved By |
| --- | --- | --- | --- |
| x0 | zero | Hardwired zero — reads always return 0, writes discarded | — |
| x1 | ra | Return address | Caller |
| x2 | sp | Stack pointer | Callee |
| x3 | gp | Global pointer | — |
| x4 | tp | Thread pointer (TLS base) | — |
| x5–x7 | t0–t2 | Temporaries | Caller |
| x8 | s0/fp | Saved / frame pointer | Callee |
| x9 | s1 | Saved register | Callee |
| x10–x11 | a0–a1 | Arguments / return values | Caller |
| x12–x17 | a2–a7 | Arguments | Caller |
| x18–x27 | s2–s11 | Saved registers | Callee |
| x28–x31 | t3–t6 | Temporaries | Caller |
| f0–f7 | ft0–ft7 | FP temporaries | Caller |
| f8–f9 | fs0–fs1 | FP saved | Callee |
| f10–f11 | fa0–fa1 | FP args / return | Caller |
| f12–f17 | fa2–fa7 | FP arguments | Caller |
| f18–f27 | fs2–fs11 | FP saved | Callee |
| f28–f31 | ft8–ft11 | FP temporaries | Caller |

## 5.3  Privilege Levels

| Level | Name | Description | UIOX Role |
| --- | --- | --- | --- |
| U (0) | User | Restricted — no CSR access, no privileged instructions | User tasks |
| S (1) | Supervisor | Kernel mode — page tables, SBI calls, trap handling | UIOX kernel |
| H (2) | Hypervisor | Optional — nested virtualisation (H extension) | Not used |
| M (3) | Machine | Full hardware control — resets, interrupt delegation | Firmware/SBI |

## 5.4  Core Instruction Set (RV64I)

### 5.4.1  Integer ALU

| Instruction | Syntax | Operation |
| --- | --- | --- |
| ADD | ADD rd, rs1, rs2 | rd ← rs1 + rs2 |
| ADDW | ADDW rd, rs1, rs2 | rd ← sext32(rs1[31:0] + rs2[31:0]) — 32-bit result sign-extended |
| ADDI | ADDI rd, rs1, imm12 | rd ← rs1 + sext(imm12) |
| ADDIW | ADDIW rd, rs1, imm | rd ← sext32(rs1[31:0] + sext(imm)) |
| SUB | SUB rd, rs1, rs2 | rd ← rs1 − rs2 |
| SUBW | SUBW rd, rs1, rs2 | rd ← sext32(rs1 − rs2) |
| AND | AND rd, rs1, rs2 | rd ← rs1 & rs2 |
| ANDI | ANDI rd, rs1, imm | rd ← rs1 & sext(imm12) |
| OR | OR rd, rs1, rs2 | rd ← rs1 ¦ rs2 |
| ORI | ORI rd, rs1, imm | rd ← rs1 ¦ sext(imm12) |
| XOR | XOR rd, rs1, rs2 | rd ← rs1 ^ rs2 |
| XORI | XORI rd, rs1, imm | rd ← rs1 ^ sext(imm12) |
| SLL | SLL rd, rs1, rs2 | rd ← rs1 << rs2[5:0] |
| SLLI | SLLI rd, rs1, shamt | rd ← rs1 << shamt |
| SRL | SRL rd, rs1, rs2 | rd ← rs1 >> rs2[5:0] (logical) |
| SRLI | SRLI rd, rs1, shamt | rd ← rs1 >> shamt (logical) |
| SRA | SRA rd, rs1, rs2 | rd ← rs1 >> rs2[5:0] (arithmetic) |
| SRAI | SRAI rd, rs1, shamt | rd ← rs1 >> shamt (arithmetic) |
| LUI | LUI rd, imm20 | rd ← imm20 << 12 (load upper immediate) |
| AUIPC | AUIPC rd, imm20 | rd ← PC + (imm20 << 12) |
| SLT | SLT rd, rs1, rs2 | rd ← (rs1 < rs2) ? 1 : 0 (signed) |
| SLTU | SLTU rd, rs1, rs2 | rd ← (rs1 < rs2) ? 1 : 0 (unsigned) |
| SLTI | SLTI rd, rs1, imm | rd ← (rs1 < sext(imm)) ? 1 : 0 (signed) |

### 5.4.2  Load / Store

| Instruction | Syntax | Operation |
| --- | --- | --- |
| LB | LB rd, off(rs1) | rd ← sext(Mem8[rs1+off]) |
| LBU | LBU rd, off(rs1) | rd ← zext(Mem8[rs1+off]) |
| LH | LH rd, off(rs1) | rd ← sext(Mem16[rs1+off]) |
| LHU | LHU rd, off(rs1) | rd ← zext(Mem16[rs1+off]) |
| LW | LW rd, off(rs1) | rd ← sext(Mem32[rs1+off]) |
| LWU | LWU rd, off(rs1) | rd ← zext(Mem32[rs1+off]) — RV64 only |
| LD | LD rd, off(rs1) | rd ← Mem64[rs1+off] — RV64 only |
| SB | SB rs2, off(rs1) | Mem8[rs1+off] ← rs2[7:0] |
| SH | SH rs2, off(rs1) | Mem16[rs1+off] ← rs2[15:0] |
| SW | SW rs2, off(rs1) | Mem32[rs1+off] ← rs2[31:0] |
| SD | SD rs2, off(rs1) | Mem64[rs1+off] ← rs2 — RV64 only |

### 5.4.3  Control Flow

| Instruction | Syntax | Operation |
| --- | --- | --- |
| BEQ | BEQ rs1, rs2, off | PC ← PC+sext(off) if rs1 == rs2 (±4 KB range) |
| BNE | BNE rs1, rs2, off | PC ← PC+sext(off) if rs1 != rs2 |
| BLT | BLT rs1, rs2, off | Branch if rs1 < rs2 (signed) |
| BGE | BGE rs1, rs2, off | Branch if rs1 >= rs2 (signed) |
| BLTU | BLTU rs1, rs2, off | Branch if rs1 < rs2 (unsigned) |
| BGEU | BGEU rs1, rs2, off | Branch if rs1 >= rs2 (unsigned) |
| JAL | JAL rd, off | rd ← PC+4; PC ← PC + sext(off20) (±1 MB) |
| JALR | JALR rd, rs1, off | rd ← PC+4; PC ← (rs1+sext(off)) & ~1 |

### 5.4.4  M Extension — Multiply / Divide

| Instruction | Operation |
| --- | --- |
| MUL rd,rs1,rs2 | rd ← lower 64 bits of rs1 × rs2 |
| MULH rd,rs1,rs2 | rd ← upper 64 bits of rs1 × rs2 (signed × signed) |
| MULHU rd,rs1,rs2 | rd ← upper 64 bits (unsigned × unsigned) |
| MULHSU rd,rs1,rs2 | rd ← upper 64 bits (signed × unsigned) |
| MULW rd,rs1,rs2 | rd ← sext32(rs1[31:0] × rs2[31:0]) |
| DIV rd,rs1,rs2 | rd ← rs1 / rs2 (signed; rounds toward zero) |
| DIVU rd,rs1,rs2 | rd ← rs1 / rs2 (unsigned) |
| REM rd,rs1,rs2 | rd ← rs1 % rs2 (signed remainder) |
| REMU rd,rs1,rs2 | rd ← rs1 % rs2 (unsigned remainder) |
| DIVW / REMW | 32-bit word versions of DIV/REM (sign-extended to 64 bits) |

### 5.4.5  A Extension — Atomics

RISC-V atomics use two mechanisms: LR/SC (Load-Reserved / Store-Conditional) for compare-and-swap loops, and AMO (Atomic Memory Operations) for direct read-modify-write. Both carry ordering annotations: no suffix = relaxed; .aq = acquire; .rl = release; .aqrl = sequentially consistent.

| Instruction | Syntax | Operation |
| --- | --- | --- |
| LR.D | LR.D{.aq} rd, (rs1) | rd ← Mem64[rs1]; set exclusive reservation |
| SC.D | SC.D{.rl} rd, rs2, (rs1) | if reservation valid: Mem64[rs1]←rs2; rd←0; else rd←1 |
| AMOSWAP.D | AMOSWAP.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] ← rs2 |
| AMOADD.D | AMOADD.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] += rs2 |
| AMOAND.D | AMOAND.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] &= rs2 |
| AMOOR.D | AMOOR.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] ¦= rs2 |
| AMOXOR.D | AMOXOR.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] ^= rs2 |
| AMOMAX.D | AMOMAX.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] ← max(rd,rs2) signed |
| AMOMAXU.D | AMOMAXU.D rd, rs2, (rs1) | rd ← Mem64[rs1]; Mem64[rs1] ← max(rd,rs2) unsigned |
| AMOMIN.D | AMOMIN.D rd, rs2, (rs1) | min (signed) |
| AMOMINU.D | AMOMINU.D rd, rs2, (rs1) | min (unsigned) |

## 5.5  CSR Instructions (Zicsr Extension)

Control and Status Registers (CSRs) are the privileged register file in RISC-V. They hold machine configuration, trap vectors, timers, performance counters, and physical memory protection settings. All CSR access is through dedicated instructions — there is no direct memory mapping.

| Instruction | Syntax | Operation |
| --- | --- | --- |
| CSRRW | CSRRW rd, csr, rs1 | rd ← csr; csr ← rs1 (read/write) |
| CSRRS | CSRRS rd, csr, rs1 | rd ← csr; csr ¦= rs1 (read/set bits) |
| CSRRC | CSRRC rd, csr, rs1 | rd ← csr; csr &= ~rs1 (read/clear bits) |
| CSRRWI | CSRRWI rd, csr, uimm5 | rd ← csr; csr ← uimm5 (immediate write) |
| CSRRSI | CSRRSI rd, csr, uimm5 | rd ← csr; csr ¦= uimm5 |
| CSRRCI | CSRRCI rd, csr, uimm5 | rd ← csr; csr &= ~uimm5 |

## 5.6  Key CSRs Used in UIOX

| CSR | Address | Privilege | Purpose |
| --- | --- | --- | --- |
| mstatus | 0x300 | M | Global interrupt enable (MIE/SIE), privilege mode bits, FP/extension state |
| misa | 0x301 | M | Supported ISA extensions (read-only) |
| mtvec | 0x305 | M | Machine trap-handler base address + mode |
| medeleg | 0x302 | M | Exception delegation to S-mode |
| mideleg | 0x303 | M | Interrupt delegation to S-mode |
| mip | 0x344 | M | Machine interrupt pending (MTIP, MSIP, MEIP) |
| mie | 0x304 | M | Machine interrupt enable |
| mepc | 0x341 | M | Machine exception PC (return address on MRET) |
| mcause | 0x342 | M | Machine trap cause (interrupt bit + cause code) |
| mtval | 0x343 | M | Machine trap value (faulting address or instruction) |
| mscratch | 0x340 | M | Scratch register for M-mode trap handler |
| satp | 0x180 | S | Supervisor address translation (page table base + ASID + mode: Sv39/Sv48) |
| stvec | 0x105 | S | Supervisor trap vector |
| sepc | 0x141 | S | Supervisor exception PC |
| scause | 0x142 | S | Supervisor trap cause |
| stval | 0x143 | S | Supervisor trap value |
| sstatus | 0x100 | S | Supervisor status (subset of mstatus) |
| sscratch | 0x140 | S | Scratch register for S-mode trap handler |
| sie | 0x104 | S | Supervisor interrupt enable |
| sip | 0x144 | S | Supervisor interrupt pending |
| time | 0xC01 | U | Real-time clock (µs resolution on most platforms) |
| cycle | 0xC00 | U | Cycle counter |
| instret | 0xC02 | U | Instructions-retired counter |

## 5.7  RISC-V Memory Model & Fences

| Instruction | Syntax | Operation |
| --- | --- | --- |
| FENCE | FENCE pred, succ | Order memory accesses: predecessor set (pred) before successor set (succ). pred/succ = any combo of I (instruction fetch), O (output), R (read), W (write). FENCE RW,RW is the full barrier. |
| FENCE.I | FENCE.I | Synchronise instruction and data streams — required before executing newly-written code (I-cache coherence). From the Zifencei extension. |
| SFENCE.VMA | SFENCE.VMA rs1,rs2 | Flush TLB for address rs1 / ASID rs2 (S-mode; rs1=x0 → all addresses; rs2=x0 → all ASIDs) |

## 5.8  SBI (Supervisor Binary Interface) Calls

RISC-V systems use the SBI to make requests from S-mode (UIOX kernel) to M-mode firmware (e.g. OpenSBI). SBI calls use ECALL with a7 = Extension ID, a6 = Function ID, and a0–a5 as arguments. Return values are in a0 (error) and a1 (value).

| EID (a7) | Extension | Key Functions |
| --- | --- | --- |
| 0x00 | Legacy Set Timer | Set next timer interrupt (replaced by EID 0x54494D45) |
| 0x01 | Legacy Console Putchar | Single-character console output |
| 0x10 | Base Extension | sbi_get_spec_version, sbi_probe_extension |
| 0x54494D45 | Timer (TIME) | sbi_set_timer(stime_value) — UIOX timer interrupt |
| 0x735049 | IPI | sbi_send_ipi(hart_mask) — inter-processor interrupt |
| 0x52464E43 | RFENCE | sbi_remote_fence_i, sbi_remote_sfence_vma |
| 0x48534D | HSM (Hart State) | sbi_hart_start, sbi_hart_stop, sbi_hart_get_status |
| 0x53525354 | SRST (System Reset) | sbi_system_reset(type, reason) — shutdown/reboot |

## 5.9  UIOX RISC-V Inline Assembly Patterns

Read time CSR (uiox_sec_plat_time_ms):

__asm__ volatile("rdtime %0" : "=r"(t));

Read cycle counter:

__asm__ volatile("rdcycle %0" : "=r"(cycles));

Full memory barrier:

__asm__ volatile("fence rw, rw" ::: "memory");

I-cache fence (after writing executable code):

__asm__ volatile("fence.i" ::: "memory");

Disable S-mode interrupts (clear SIE in sstatus):

__asm__ volatile("csrc sstatus, %0" :: "r"(0x2u) : "memory");

Enable S-mode interrupts:

__asm__ volatile("csrs sstatus, %0" :: "r"(0x2u) : "memory");

Read satp (page table base):

__asm__ volatile("csrr %0, satp" : "=r"(satp_val) :: "memory");

Write satp and flush TLB:

__asm__ volatile("csrw satp, %0 \n\t" "sfence.vma" :: "r"(satp_val) : "memory");

SBI set-timer call (EID=0x54494D45, FID=0):

register long a7 __asm__("a7") = 0x54494D45L;

register long a6 __asm__("a6") = 0L;

register long a0 __asm__("a0") = (long)stime;

__asm__ volatile("ecall" : "+r"(a0) : "r"(a6), "r"(a7) : "memory");

# 6 · Cross-Architecture Comparison

## 6.1  ISA Characteristics

| Property | ARM32 | ARM64 | x86-64 | RISC-V |
| --- | --- | --- | --- | --- |
| ISA family | CISC-leaning RISC | RISC | CISC | Pure RISC |
| Instruction width | 32-bit (Thumb: 16/32) | Fixed 32-bit | Variable 1–15 bytes | 32-bit (C ext: 16-bit) |
| GPRs | 16 × 32-bit | 31 × 64-bit | 16 × 64-bit | 32 × 64-bit |
| Condition codes | Per-instruction | NZCV flags reg | RFLAGS | No flags (branch compares) |
| Calling convention | AAPCS (R0–R3 args) | AAPCS64 (X0–X7) | SysV AMD64 (RDI/RSI…) | RISC-V psABI (a0–a7) |
| Syscall instruction | SVC #0 | SVC #0 | SYSCALL | ECALL |
| Kernel entry reg | R7 (Linux) | X8 (number) | RAX (number) | a7 (EID) + a6 (FID) |
| Page granule | 4 KB (LPAE: 4/64K) | 4 KB / 16 KB / 64 KB | 4 KB / 2 MB / 1 GB | 4 KB (Sv32/Sv39/Sv48/Sv57) |
| Cache coherence | DSB/DMB/ISB | DSB/DMB/ISB | MFENCE/SFENCE/LFENCE | FENCE / FENCE.I |
| Hardware TRNG | SMCCC TRNG / MMIO | RNDR + SMCCC | RDRAND + RDSEED | SBI / MMIO |
| Atomic model | LDREX/STREX + LDAEX | LDXR/STXR + LSE | LOCK prefix + CMPXCHG | LR/SC + AMO (.aq/.rl) |

## 6.2  UIOX Integration Summary

| Subsystem | ARM32 | ARM64 | x86-64 | RISC-V |
| --- | --- | --- | --- | --- |
| arch_init source | 10_Arch/arm32/src/arch_init.c | 10_Arch/arm64/src/arch_init.c | 10_Arch/x86_64/src/arch_init.c | 10_Arch/riscv/src/arch_init.c |
| Timer source | SP804 MMIO + CP15 CNTPCT | ARM Generic Timer CNTPCT_EL0 | TSC (RDTSC) | rdtime CSR / SBI SET_TIMER |
| IRQ controller | GIC-400 MMIO | GIC-400 / GIC-v3 sysreg | LAPIC + IOAPIC | PLIC (Platform-Level Interrupt Controller) |
| UART | PL011 @ 0x10009000 | PL011 @ 0x09000000 | 16550 port I/O or MMIO | SBI console / UART MMIO |
| TRNG BSP | bsp/uiox_sec_plat_arm32.c | bsp/uiox_sec_plat_arm64.c | bsp/uiox_sec_plat_x86_64.c | (future) bsp/uiox_sec_plat_riscv.c |
| MMU type | ARMv7 Short / LPAE | AArch64 stage-1 VMSAv8 | x86-64 4-level paging | Sv39 / Sv48 |
| Compiler target | -march=armv7-a -mfloat-abi=hard | -march=armv8-a | -march=x86-64 | -march=rv64imafdc |

# 7 · Appendix

## 7.1  Condition Code Quick Reference (ARM32 / ARM64)

| Code | Meaning | CPSR/NZCV Condition |
| --- | --- | --- |
| EQ | Equal | Z=1 |
| NE | Not Equal | Z=0 |
| CS/HS | Carry Set / Unsigned Higher or Same | C=1 |
| CC/LO | Carry Clear / Unsigned Lower | C=0 |
| MI | Minus / Negative | N=1 |
| PL | Plus / Non-negative | N=0 |
| VS | Overflow Set | V=1 |
| VC | Overflow Clear | V=0 |
| HI | Unsigned Higher | C=1 && Z=0 |
| LS | Unsigned Lower or Same | C=0 ¦¦ Z=1 |
| GE | Signed Greater or Equal | N=V |
| LT | Signed Less Than | N≠V |
| GT | Signed Greater Than | Z=0 && N=V |
| LE | Signed Less or Equal | Z=1 ¦¦ N≠V |
| AL | Always (default) | (any) |

## 7.2  x86-64 Common Jcc Mnemonics

| Mnemonic | Aliases | Jumps When |
| --- | --- | --- |
| JE | JZ | ZF=1 (equal / zero) |
| JNE | JNZ | ZF=0 (not equal / not zero) |
| JG | JNLE | ZF=0 && SF=OF (signed greater) |
| JGE | JNL | SF=OF (signed greater or equal) |
| JL | JNGE | SF≠OF (signed less) |
| JLE | JNG | ZF=1 ¦¦ SF≠OF (signed less or equal) |
| JA | JNBE | CF=0 && ZF=0 (unsigned above) |
| JAE | JNB, JNC | CF=0 (unsigned above or equal) |
| JB | JNAE, JC | CF=1 (unsigned below) |
| JBE | JNA | CF=1 ¦¦ ZF=1 (unsigned below or equal) |
| JS | — | SF=1 (negative) |
| JNS | — | SF=0 (non-negative) |
| JO | — | OF=1 (overflow) |
| JNO | — | OF=0 (no overflow) |

## 7.3  RISC-V mcause / scause Values

| Interrupt bit | Code | Cause |
| --- | --- | --- |
| 1 (interrupt) | 1 | Supervisor software interrupt |
| 1 | 3 | Machine software interrupt |
| 1 | 5 | Supervisor timer interrupt |
| 1 | 7 | Machine timer interrupt |
| 1 | 9 | Supervisor external interrupt (PLIC) |
| 1 | 11 | Machine external interrupt |
| 0 (exception) | 0 | Instruction address misaligned |
| 0 | 1 | Instruction access fault |
| 0 | 2 | Illegal instruction |
| 0 | 3 | Breakpoint (EBREAK) |
| 0 | 4 | Load address misaligned |
| 0 | 5 | Load access fault |
| 0 | 6 | Store/AMO address misaligned |
| 0 | 7 | Store/AMO access fault |
| 0 | 8 | Environment call from U-mode (ECALL) |
| 0 | 9 | Environment call from S-mode |
| 0 | 11 | Environment call from M-mode |
| 0 | 12 | Instruction page fault |
| 0 | 13 | Load page fault |
| 0 | 15 | Store/AMO page fault |

## 7.4  References

- ARM Architecture Reference Manual — ARMv7-A and ARMv7-R edition (ARM DDI 0406)

- ARM Architecture Reference Manual — ARMv8-A (ARM DDI 0487)

- ARM SMCCC Specification — DEN0028 / DEN0098

- Intel 64 and IA-32 Architectures Software Developer's Manual, Vol. 1–3

- AMD64 Architecture Programmer's Manual, Vol. 1–5

- RISC-V Instruction Set Manual, Vol. I (Unprivileged), Vol. II (Privileged) — riscv.org

- RISC-V SBI Specification — github.com/riscv-non-isa/riscv-sbi-doc

- UIOX Repository — github.com/Pramod645/UIOX (branch UIX00)

Generated by UIOX documentation toolchain · 2026-07-12